PRACTICAL LAB - 2

DIGITAL LOGIC DESIGN

Assign by: SIR ABDUL RAFAY

Prepared by: Muhammad Aamir

1.  Verify following Rules of Boolean Algebra by designing them using Circuit Makers.

Identity Law:

Idempotent Law:

Complement Law:

Involution Law:

Commutative Law:

Associative Law:

Distributive Law:

2. Simplify the expressions  1. F=(A+(BC)’)’

Ans:

F = [ A ] ’  [ ( B C ) ’ ] ’

F=  A ’ B C

2. F=(AB+CD)’

Ans:

F =  ( A B ) ’  ( C D ) ’

3. Use DE Morgan’s Theorem to prove that this NAND gate circuit performs the exact same function:

Sol:

# F = [(AB)’ (CD)’]’             //BREAKING LONGEST COMPLIMENT

F = [(AB’)’ + [(CD)’]’     //COMPLIMENT LAW

F = AB + CD

4. Apply the principles of DE Morgan’s theorems to the simplification of a gate circuit:

SOL:

# F = [{A (B+C)}’ + (AB’)’]’      //BREAKING LONGEST COMPLIMENT

F = [A (B+C)] . (AB’)             // DISTRIBUTIVE LAW

F = (AB + AC) . AB’              // MULTIPLYING

F = (AB’) (AB) + (AB’) (AC)      //A.A=A , B.B’=0

F= A(0) + AB’C

F= AB’C